SIC SOFTWARE TEST RUN

The following procedure assembles, loads, and executes a simple SIC

program using the SIC simulator. This is not an exhaustive test of

the SIC software supplied in this archive. However, successfully

completing this procedure should provide reasonable assurance that

everything has been installed properly. (This procedure can be modified,

as described below, to test only the simulator, bootstrap, and loader

in case you prefer not to install the assembler.)

The SIC program that is assembled and executed during this procedure

copies a file from device F3 to device 06. This program is a

modification of the one that appears in Fig. 2.5 of "System Software."

The main difference is that this version writes end-of-line

indications as part of its output (by considering the end-of-line

as a part of the record).

1. Install the SIC simulator and (if desired) the SIC assembler.

2. If you do NOT wish to install the SIC assembler, place the

contents of SICTEST.OBJ into a file named DEVF2 and go to

step 5 below.

3. Copy the contents of SICTEST.ASM into a file named SRCFILE

and execute the SIC assembler.

4. Copy the contents of OBJFILE into a file named DEVF2. (This

object program should be the same as the one found in

SICTEST.OBJ).

5. Copy the contents of SICBOOT.OBJ into a file named DEV00.

6. Copy the contents of SICLDR.OBJ into a file named DEVF1.

7. Copy the contents of SICTEST.DAT into a file named DEVF3.

8. Execute the SIC simulator, entering commands as shown in the

simulation log below.

9. Examine the contents of file DEV06. This file should contain

the following seven lines:

TEST LINE 1

TEST LINE 2

3

4

5

END OF TEST FILE

EOF

The following listing shows all of the input and output messages that

should occur during the simulator run:

SIC SIMULATOR V1.6

COMMAND: S(tart, R(un, E(nter, D(ump, H(count, B(kpt, Q(uit?

D R,0-8F

A=FFFFFF  X=FFFFFF  L=FFFFFF  B=FFFFFF

S=FFFFFF  T=FFFFFF  P=000000  CC=LT

0000 FFFFFFFF FFFFFFFF FFFFFFFF FFFFFFFF

0010 FFFFFFFF FFFFFFFF FFFFFFFF FFFFFFFF

0020 FFFFFFFF FFFFFFFF FFFFFFFF FFFFFFFF

0030 FFFFFFFF FFFFFFFF FFFFFFFF FFFFFFFF

0040 FFFFFFFF FFFFFFFF FFFFFFFF FFFFFFFF

0050 FFFFFFFF FFFFFFFF FFFFFFFF FFFFFFFF

0060 FFFFFFFF FFFFFFFF FFFFFFFF FFFFFFFF

0070 FFFFFFFF FFFFFFFF FFFFFFFF FFFFFFFF

0080 FFFFFFFF FFFFFFFF FFFFFFFF FFFFFFFF

COMMAND: S(tart, R(un, E(nter, D(ump, H(count, B(kpt, Q(uit?

S

COMMAND: S(tart, R(un, E(nter, D(ump, H(count, B(kpt, Q(uit?

D R,0-8F

A=FFFFFF  X=FFFFFF  L=FFFFFF  B=FFFFFF

S=FFFFFF  T=FFFFFF  P=000000  CC=LT

0000 00004B04 00484800 1E200054 0C005848

0010 001E1800 58548000 2C004B3C 0006E000

0020 5730001E D8005728 00423000 80280045

0030 38001E1C 00452800 5138003F 1C004E4C

0040 00000000 04000030 00008000 00000000

0050 0700000A 000010F1 00000000 00000000

0060 00000000 00000000 00000000 00000000

0070 00000000 00000000 00000000 00000000

0080 FFFFFFFF FFFFFFFF FFFFFFFF FFFFFFFF

COMMAND: S(tart, R(un, E(nter, D(ump, H(count, B(kpt, Q(uit?

B 1027

COMMAND: S(tart, R(un, E(nter, D(ump, H(count, B(kpt, Q(uit?

H 5000

COMMAND: S(tart, R(un, E(nter, D(ump, H(count, B(kpt, Q(uit?

R

5000 INSTRUCTIONS EXECUTED

P=000030

COMMAND: S(tart, R(un, E(nter, D(ump, H(count, B(kpt, Q(uit?

R

5000 INSTRUCTIONS EXECUTED

P=00001B

COMMAND: S(tart, R(un, E(nter, D(ump, H(count, B(kpt, Q(uit?

R

5000 INSTRUCTIONS EXECUTED

P=000115

COMMAND: S(tart, R(un, E(nter, D(ump, H(count, B(kpt, Q(uit?

R

5000 INSTRUCTIONS EXECUTED

P=002079

COMMAND: S(tart, R(un, E(nter, D(ump, H(count, B(kpt, Q(uit?

R

BREAKPOINT REACHED

P=001027

COMMAND: S(tart, R(un, E(nter, D(ump, H(count, B(kpt, Q(uit?

D R,0-8F

A=000046  X=000003  L=0000DF  B=FFFFFF

S=FFFFFF  T=FFFFFF  P=001027  CC=EQ

0000 00004B04 00484800 1E200054 0C005848

0010 001E1800 58548000 2C004B3C 0006E000

0020 5730001E D8005728 00423000 80280045

0030 38001E1C 00452800 5138003F 1C004E4C

0040 00000000 04000030 00008000 00000000

0050 0700000A 000010F1 0000F000 00000000

0060 00000000 00000000 00000000 00000000

0070 00000000 00000000 00000000 00000000

0080 48010C28 01623000 8AFF4801 4D48010C

COMMAND: S(tart, R(un, E(nter, D(ump, H(count, B(kpt, Q(uit?

Q